C:\Users\HH\Documents\CENTC276\ISO TC 91 WG3\2019-09-25 ISOTC91WG3 MADRID\ISO_TR_21681_(E) first draft proposal.docxWORKING DRAFT© ISO 2019 – All rights reservedISO/PDTR 21681:2019(E) 63Overview on surfactants containing biomassSurface active agents — Bio-based surfactantsÉlément introductif — Élément central — Élément complémentaireSurface active agents — Bio-based surfactants — Overview on surfactants containing biomassE2019-09-25(20) PreparatoryISOISO/   Technical Report201926ISO/TR 21681ISO/TR 21681ISO/PDTR 21681 ISIRI Surface active agents3 91 2Titre 2Titre 1     0 STD Version 2.8f10   4             ISO/TC 91

Date:   2020-03-05

ISO/DTR 21681

Secretariat:   ISIRI

Surface active agents — Bio-based surfactants — Overview on surfactants containing biomass

Élément introductif — Élément central — Élément complémentaire

Warning

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

Copyright notice

This ISO document is a working draft or committee draft and is copyright-protected by ISO. While the reproduction of working drafts or committee drafts in any form for use by participants in the ISO standards development process is permitted without prior permission from ISO, neither this document nor any extract from it may be reproduced, stored or transmitted in any form for any other purpose without prior written permission from ISO.

Requests for permission to reproduce this document for the purpose of selling it should be addressed as shown below or to ISO's member body in the country of the requester:

[Indicate the full address, telephone number, fax number, telex number, and electronic mail address, as appropriate, of the Copyright Manager of the ISO member body responsible for the secretariat of the TC or SC within the framework of which the working document has been prepared.]

Reproduction for sales purposes may be subject to royalty payments or a licensing agreement.

Violators may be prosecuted.

Contents 	Page

Foreword	iv

Introduction	v

1	Scope	1

2	Normative references	1

3	Terms and definitions	1

4	Market situation and penetration of bio-based surfactants in Europe	2

5	Description of differences between bio-based and non-bio-based surfactant	4

5.1	Impact on biodegradability/aquatic toxicity	4

5.2	Performance	5

5.3	Life Cycle Assessment LCA and Product Environmental Footprint PEF [3]	6

5.4	Sustainability	7

6	Characterization of sources	9

6.1	Specific methods	9

6.2	Use and impact	9

6.3	Round robin and Proficiency test:	11

6.4	Conclusions of this round robin	13

7	Labelling and certification schemes	14

Bibliography	15

Foreword

ISO (the International Organization for Standardization) is a worldwide federation of national standards bodies (ISO member bodies). The work of preparing International Standards is normally carried out through ISO technical committees. Each member body interested in a subject for which a technical committee has been established has the right to be represented on that committee. International organizations, governmental and non-governmental, in liaison with ISO, also take part in the work. ISO collaborates closely with the International Electrotechnical Commission (IEC) on all matters of electrotechnical standardization.

The procedures used to develop this document and those intended for its further maintenance are described in the ISO/IEC Directives, Part 1. In particular the different approval criteria needed for the different types of ISO documents should be noted. This document was drafted in accordance with the editorial rules of the ISO/IEC Directives, Part 2 (see www.iso.org/directives).

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights. ISO shall not be held responsible for identifying any or all such patent rights. Details of any patent rights identified during the development of the document will be in the Introduction and/or on the ISO list of patent declarations received (see www.iso.org/patents).

Any trade name used in this document is information given for the convenience of users and does not constitute an endorsement.

For an explanation on the meaning of ISO specific terms and expressions related to conformity assessment, as well as information about ISO's adherence to the World Trade Organization (WTO) principles in the Technical Barriers to Trade (TBT) see the following URL: www.iso.org/iso/foreword.html.

The committee responsible for this document is ISO/XXX

This second/third/... edition cancels and replaces the first/second/... edition (), [clause(s) / subclause(s) / table(s) / figure(s) / annex(es)] of which [has / have] been technically revised.

ISO XXXX consists of the following parts. [Add information as necessary.]

Introduction

This document is a working document.

Bio-based raw materials have been used for millennia in the manufacture of surfactants, e.g. the first surfactant used by mankind was already completely bio-based – soap. With the advent of modern surfactants in the early 20th Century, petrochemical-based raw materials also became of interest. They offered the opportunity to tune the surfactant properties, in a broader sense, to their various applications.

The last decades have seen the emergence of new bio-based raw materials for surfactants. Some of the reasons for the increased interest lie in the bio-based products’ potential benefits in relation to the depletion of fossil resources and climate change.

Acknowledging the need for common standards for bio-based products, the European Commission issued mandate M/492, resulting in a series of standards developed by CEN/TC 411 and now developed in ISO TC 91, with a focus on bio-based products other than food, feed and biomass for energy applications.

The standards on "Bio-based products" provide a common basis on the following aspects:

Common terminology

Bio-based content determination

Life Cycle Assessment (LCA)

Sustainability aspects

Declaration tools.

It is important to understand what the term “bio-based product” covers and how it is being used. The term "bio-based" means "derived wholly or partly from biomass". It is essential to characterise the amount of biomass contained in the product by, for instance, its (total) bio-based content or bio-based carbon content.

The bio-based content of a product itself does not provide information on its environmental impact or sustainability, which may be assessed through Life Cycle Inventory (LCI), LCA and sustainability criteria. In addition, transparent and unambiguous communication within bio-based value chains is facilitated by a harmonised framework for certification and declaration.

Surfactants are products which have the ability to reduce interfacial/surface tension, wet surfaces, suspend materials or emulsify oils and fats. In Europe, thousands of producers, manufacturers and nearly every inhabitant in Europe use surfactants every day in consumer or industrial applications. The surfactant-producing industry is composed of mainly multinationals. Downstream users are found in multinationals as well as SME's.

Surfactants may be produced from both fossil and renewable carbon feedstock (Ref. EN 16575:2014 nomenclature). The amount of crude oil used for surfactant production is, however, low with less than 1% of the total world's crude oil consumption.

Finally, the approach for these Technical Reports/Specifications/Standards intends to strengthen and harmonise the reputation of "bio-based surfactants” and the confidence of the customer in this product group.

Surface active agents — Bio-based surfactants — Overview on surfactants containing biomass

# Scope

The aim of this technical report is to summarise the actual situation regarding many aspects regarding bio-based surfactants and their relation to any other surfactant regardless of its origin.  It will describe existing raw material sources with regard to their current usage in surface active agents, their source identification and conformation, and the options for communication same.

It shall also include the current work on surfactants regarding their performances, their sustainability, the LCA approaches and end of life options.

# Normative references

The is no normative reference in this report

# Terms and definitions

For the purposes of this document, the terms and definitions given in ISO 21680 and the following apply.

3.1

surfactant

organic substance, possessing surface activity which, dissolved in a liquid, particularly water, lowers the surface or interfacial tension, by preferred adsorption at the liquid/vapour surface, or other interfaces

Note 1 to entry:	"substance" as defined in REACH [2]

[SOURCE: ISO 862:1984, surface active agent 1.1 modified]

3.2

bio-based surfactant

surfactant wholly or partly derived from biomass (based on biogenic carbon) produced   either  by  chemical or biotechnological processing

[SOURCE: EN 16575:2019, bio-based surfactant 3.2]

3.3

bio surfactant

surfactant   wholly  based  on   biomass   (based   on  biogenic  carbon)  produced   either  by  chemical or biotechnological processing

[SOURCE: EN 16575:2019, bio-surfactant 3.3]

3.4

degradation

transformation of a compound into smaller component parts due to the physico-chemical environment. This may occur due to abiotic processes such as oxidation and UV adsorption.

[SOURCE: EN 16575:2019, degradation 3.4]

3.5

biodegradation

transformation of a compound into smaller component parts due to biological processes,

[SOURCE: EN 16575:2019, biodegradation 3.5]

3.6

ultimate biodegradation

breakdown of organic matter by micro-organisms in the presence of oxygen to carbon dioxide, water and mineral salts of any other elements present (mineralisation) or in absence of oxygen to carbon dioxide, methane and mineral salts, and in both cases the production of new biomass.

[SOURCE: EN 16575:2019, ultimate biodegradation 3.6]

# Market situation and penetration of bio-based surfactants in Europe

Surfactants consist of at least one hydrophobic and one hydrophilic part. The source of these parts can be either fossil-based or bio-based (renewable).

Traditionally, triglycerides from various oil plants are used as renewable sources for the hydrophobic part. Suitable plants are e.g. oil palms, coconut palms, sunflowers, rapeseeds, or soy beans. The oils taken from these plants are further chemically processed to get fatty acids by saponification or to get fatty alcohols by methanolysis of the triglycerides and consecutive hydrogenation of the fatty methylester.

If fossil resources are used for the hydrophobic part, there is a multitude of pathways to process either crude oil or natural gas to build the hydrophobic building block. This includes e.g. Fischer-Tropsch synthesis, oxo process, olefin oligomerisation, or Friedel-Crafts alkylation.

Generally, the carbon number of the hydrophobic part of a surfactant ranges from 4 to 24 carbon atoms, whereas the majority has a carbon number between 10 and 15 carbon atoms.

Also the hydrophilic part may be derived from different sources either organic from renewable or fossil sources or inorganic from minerals. Table 1 gives a non-exhaustive summary.

Table 1 — Hydrophilic components of surfactants and their origin

| Hydrophilic source | Type of material |
| --- | --- |
| Renewable | Sugar, starch |
| Fossil | EO, chloroacetic acid |
| Inorganic | Sulfate, sulfonate, phosphate |

The production of surfactants is carried out by multinational companies as well as SME’s. Figure1 shows the use of bio-based and non bio-based surfactants in EU (+ Norway, Iceland, and Switzerland) in 2015. The split is close to be equal between them. Partitioning the bio-based part further into minority bio-based surfactants (5%< bio-based carbon content ≤50%), majority bio-based surfactants (50% < bio-based carbon content <95%), and fully bio-based surfactants (bio-based carbon content ≥95%) show further details of the use of the different types of surfactants.

Keys

  - FBB = Fully Bio-Based (3%)

  - MaBB = Majority Bio-Based (30%)

  - MiBB = Minority Bio-Based (19%)

  - NBB = Non Bio-Based (48%)

Figure 1 —Proportion of bio-based and non bio-based surfactants used in the European Union + Norway, Switzerland, and Iceland in 2015 (Source: CESIO 2016)

# Description of differences between bio-based and non-bio-based surfactant

## Impact on biodegradability/aquatic toxicity

Surface active agents are generally discharged via treated and untreated wastewater into the environment. Consequently, ultimate biodegradability (mineralisation) is often required for such substances. There are numerous test methods to measure biodegradability which may be used [2].

Common criteria for biodegradability is in most cases the ultimate biodegradation within 28 days and a pass level of 60% of either the theoretical consumption of oxygen or the theoretical evolution of carbon dioxide.

If these ultimate degradation methods cannot be measured because of the physical characteristics of the surfactant other methods [3] determining the elimination can be used. Please note that the high initial test concentration may be inhibitory because of toxic effects of the test substance to microorganisms.

In general, structural properties and molecular weight of any substance are decisive for their biodegradation. This is independent from the origin of the used raw materials as is shown in Table 2 where some examples are given using the methods described in [2] or [3]. Completely fossil-based surfactants like linear alkylbenzene sulfonates LAS are biodegradable and on the other hand majority bio-based surfactants like cetyltrimethylammonium bromide CTAB are not biodegradable.

Table 2 — "Examples of Biodegradability in regard of the Bio based raw material origin"

| Product group | Bio-based raw materials | Biodegradability |
| --- | --- | --- |
| Alcohol ethoxylates | mixed/no | yes |
| Alcohol alkoxylates | mixed/no | yes/no |
| Alkylphenol ethoxylates | no | no |
| EO/PO blockpolymers | no | yes/no |
| Alcohol sulfates | yes/no | yes |
| Alcohol ether sulfates | yes | yes |
| Alkyl polyglucosides | yes | yes |
| Linear alkylbenzenesulfonates | no | yes |
| Quaternary ammonium compounds (CTAB) | mixed | no |
| Quaternary ammonium compounds (Esterquats) | mixed | yes |
| Betaines | mixed | yes |

## Performance

Performance and properties of any molecule, including surfactants, is determined by its chemical structure and not by the origin of its raw materials.

Surfactants are used in such a wide variety of applications that it is not convenient to evaluate their performance with respect to each application. Therefore, a practical approach is to define a set of measurable surfactant properties which enable technical specialists to select appropriate surfactants and guide their evaluation.

Surfactants have the ability to

  - reduce interfacial/surface tension,

  - wet surfaces,

  - foam or defoam,

  - suspend materials, or to

  - emulsify oils and fats, among others.

They make it possible to process, apply, clean or separate materials. Surfactants are widely used in consumer and professional products and for industrial applications. Surfactants are typically used on their own or in combination with other surfactants and other agents to fulfil the requirements of the respective applications.

Examples of applications for surfactants are

  - cleaning agent

  - foaming/defoaming agent

  - wetting agent

  - emulsifier

  - viscosity modifier

  - surface tension reducer

  - process aid

  - fabric softener

Therefore, it is necessary to have a common set of technical properties characterising the performance of surfactants including bio-based surfactants. In the absence of international surfactant specification standards, it is necessary to provide to potential users the means to qualify the bio-based surfactant products, especially for its technical performance. There are a number of other factors which will determine the acceptance of a surfactant product such as the Health, Safety and Environmental properties which are not treated in this document.

This common set can be taken from the following list and will be described in more detail in the respective technical specification [1] and standard.

  - Solubility

  - Surface tension

  - Foaming power

  - Wetting performance

  - Emulsion power

## Life Cycle Assessment LCA and Product Environmental Footprint PEF [3]

It is important to monitor the sustainability of bio-based surfactants, especially as most of them are discarded after their use into the environment. It is also important that newly introduced bio-based surfactants do not compromise on sustainability versus already used surfactants for the respective application. Methods like Life Cycle Assessment LCA, Life Cycle Inventory LCI or Product Environmental Footprint PEF have been developed to assess the impact of any chemical product on the environment.

Suitable LCA and LCI methods to estimate and calculate the impact of products on the environment have been established by ISO under the following standards:

  - ISO 14040 Environmental management – Life cycle assessment – Principles and framework

  - ISO 14044 Environmental management – Life cycle assessment – Requirements and guidelines

Given by mandate M/491 [4] CEN/TC-411 has developed standards for life cycle assessment and sustainability for bio-based products:

  - EN 16751 Bio-based products – sustainability criteria

  - EN 16760 Bio-based product – Life Cycle Assessment

  - CEN/TR 16957 Bio-based products – Guidelines for Life Cycle Inventory (LCI) for the End-of-life phase

Furthermore, the European Commission (DG Environment) has worked together with the European Commission's Joint Research Centre and other European Commission services towards the development of a broad and harmonised methodology for the calculation of the environmental footprint of products (including carbon).

Existing methods and initiatives were considered

  - For the product angle, the International Reference Life Cycle Data System (ILCD) Handbook as well as other existing methodological standards and guidance documents (ISO 14040-44, PAS 2050, BP X30, WRI/WBCSD GHG protocol, Sustainability Consortium, ISO 14025, Ecological Footprint, etc.).

The final method, called Product Environmental Footprint (PEF), was published as an Annex to the Commission Recommendation on the use of common methods to measure and communicate the life cycle environmental performance of products and organisations. The two methods are tightly interlinked and will have many elements in common.

The PEF methodology is currently evaluated in a pilot phase including the use of (bio-based) surfactants in household detergents. The suitability of this method for bio-based surfactants cannot be determined at the current stage of development.

## Sustainability

Sustainable development is defined as development that meets the needs of the present without compromising the ability of future generations to meet their own needs (Definition in EN 16751:2016. Sustainability is based on 3 pillars:

economic,

environmental,

societal.

Bio-based surfactants, like any other surfactants, have a sustainability profile with some positive and negative impacts. The sustainability profile of bio-based surfactants depends on

the biomass used as raw materials,

the chemistry involved in the production of the bio-based surfactants. Thus, the type of biomass, the geographical origin of the biomass, the transportation mode and the manufacturing process of the bio-based surfactants contribute amongst others to the sustainability profile of the surfactants.

The mode of production of the biomass - such as responsible farming practices - impacts significantly the sustainability profile.

The following criteria impacting sustainability versus fossil raw materials are listed (non-exhaustive) in table 3:

Table 3 — criteria impacting sustainability versus fossil raw materials

|  | Positive impacts | Negative impacts |
| --- | --- | --- |
| Economic | Local economic enrichment | Climate, seasonal fluctuations on harvests impact yield and costs of biomass |
|  | Contribution to circular economy | Difficulty to predict production volume and therefore impact negatively the robustness of bio-based supply chain |
|  | Creation of additional revenues (e.g. for farmers) |  |
|  | Development of the local economy in remote areas with potential social benefits (development of access to education, health services) |  |
| Environmental | Lower CO2 emissions depend on crop type | Water use (consumption of high quantity of water) |
|  | Renewable raw materials | Intensive farming |
|  | Limitation of erosion and increase of green coverage | Loss of biodiversity (if land taken from natural land) |
|  | Positive contribution to the natural water cycle | Environmental impact on water quality in some conditions (e.g. if pesticides are used) |
|  | Valorisation of co-products and green waste | Degradation of soil quality, erosion |
|  | Limitation of environmental damages if responsible farming practices are used | Deforestation / CO2 emission from peat |
|  | Local employment, small family enterprises | Food vs. renewable raw materials |
| Societal | Innovation (new cultures, new processes, new supply chains, etc.) | Labor in developing world environments (child labour, forced labour) |
|  | Development of new types of jobs and new skills |  |

The origin of the biomass (bio-based vs. fossil-based) has no impact on the environmental fate and biodegradation of the surfactants.  Besides, there is no inherent advantage in transport costs of bio-based raw materials vs. fossil raw materials as it depends on the crop considered and the locations of the raw material production and surfactant production.

# Characterization of sources

## Specific methods

- ASTM D6866 - Standard Test Methods for Determining the Bio-based Content of Solid, Liquid, and Gaseous Samples Using Radiocarbon Analysis

- EN 16640 : 2017 - Bio-based products - Bio-based carbon content - Determination of the bio-based carbon content using the radiocarbon method

## Use and impact

Because surfactants include a wide spread of Physico-Chemical families, CEN/TC 276/WG3 experts decided to test EN 16640 Radiocarbon method on various substances (anionic, cationic, non-ionic, blends of surfactants).

What we highlighted in reading EN 16640, did not convinced us, that surfactants were clearly bio-based assessed by C14 content.

In fact, the amounts of surfactant in finished products, e.g. window cleaner, laundry detergents, can be considerably low (less than 10%) or very low (less than 1%).

This is the reason why we initiated our own determination of bio-based content in surfactants only.

Results shown in Table 4 revealed good relationship between experimental and expected values (calculated on loadings/reaction).

Table 4 —Comparison between calculated and measured values

| Surfactants | % Exp | % Bio calc |
| --- | --- | --- |
| Lactyllaktylate | 100 | 100 |
| Alkanesulfonate, Na salt | 0 | 0 |
| Amineoxide | 84,5 | 85,7 |
| Alkylethersulfate, Na salt | 34,3 | 31,6 |
| Alkyldimethylbenzylammonium chloride | 57,9 | 57,1 |
| Dialkylesterquat, ammoniumsalt* | 77,5 | 77,8 |
| Alkylbetaine | 74,5 | 76 |
| Alkylesterphosphate | 0 | 0 |
| Monoalkylglycerol | 100 | 100 |

Keys

X = calculated bio content

Y= experimental bio content

Figure 2 — Comparison between calculated and experimental bio content of surfactants

This encouraging results confirmed C14 determination can be a relevant method for bio content assessment in surfactants. The precision calculated based on this first regression approach is about

1 < Uc < 3

Where Uc is Uncertainty

Uc= 3xOrdinate/Slope.

In this case all surfactants are homogenous by itself or after heating and homogenation (if necessary, i.e. * in Table 5)

Surfactants are also available as blends, so we need to assess the suitability of the test method EN 16640 also for blends. Using different blends made of polyethylene glycol and alkylsulfate, a private laboratory based in US carried out the analytics and was not able to determine the right bio-based carbon content of the samples (see Figure 3).

Keys

  - X=Bio-based carbon calculated during formulation

  - Y=Bio-based carbon measured through AMS method

Figure 3 — Mismatch of experimental data vs. calculated bio-based carbon content in blends

In fact, the blends (including solid substances) highlighted a large discrepancy between global composition and substances distribution (lack of homogeneity of the sample). This heterogeneity, inside some samples, generate big deviation between expected to measured values. The amount required for C14 determination is so low (about 1 mg) that sample homogeneity is the key point of reliable measurement.

## Round robin and Proficiency test:

Taking this information into account, decision has been made to prepare carefully a new set of samples, but better mixed while solid non-ionic surfactants are involved. As C14 radiocarbon method demonstrated its ability to correctly assess bio-based content in surfactant, only non-ionic surfactants were submitted to 6 different laboratories over European countries.

Only five Laboratories provided results with reasonable time. It is obvious when special precautions (in sample mixing/milling) are taken that there is good fitting within values, covering the whole range of bio-content possibly.

Table 5 — List of compound (revised preparation)

| # | Compound | Formula | Bio-based Carbon content (%) | Appearance |
| --- | --- | --- | --- | --- |
| 1 | C16C18 ethoxylate (80 EO) | C177H356O81 | 9,8% | Powder |
| 2 | C12C18 ethoxylate (7 EO) | C28H58O8 | 49,8% | Cast solid |
| 3 | C12C14 aminoxide | C15H33NO | 86,6% | Aqueous liquid |
| 4 | 1:1 blend of #1+#2 | n.a. | 29,8% | Cast solid |
| 5 | 1:1 blend of #2+#3 | n.a. | 68,2% | Gel |

Table 6 — Results of samples C14 analysis (revised preparation)

| Laboratory | Sample |  |  |  |  | Response Time (days) |
| --- | --- | --- | --- | --- | --- | --- |
|  | #1 | #2 | #3 | #4 | #5 |  |
| A |  |  |  |  |  | ∞ |
| B | 10,76 | 50,01 | 88,79 | 69,09 | 31,41 | 80 |
| C | 11,04 | 49,63 | 88,66 | 69,88 | 30,71 | 29 |
| D | 10,96 | 49,74 | 88,94 | 70,15 | 30,63 | 81 |
| E | 10,9 | 49,4 | 88,93 | 68,93 | 31,43 | 65 |
| F | 10,95 | 47,93 | 88,15 | 69,42 | 30,87 | 48 |
| Mean | 10,92 | 48,22 | 88,69 | 69,49 | 31,01 |  |
| SD | 0,10 | 3,31 | 0,33 | 0,52 | 0,38 |  |
| CoV (%) | 0,95 | 6,86 | 0,37 | 0,74 | 1,24 |  |

Keys

  - X=Bio-based carbon content calculated during formulation (%)

  - Y=Bio-based carbon measured through AMS analysis (%)

Figure 4 — Match of experimental data vs. calculated bio-based carbon content in blends

## Conclusions of this round robin

Summarizing the values we get from these determinations, it tell us;

Radiocarbon C14 method is appropriate to measure Bio-Based content in surfactants

Action of homogeneity checking (on blends) must be done to ensure reliable results

Academic laboratories play an important role as quality actors in this field (especially in Europe)

General Precision (Quantification range: 5-100%) is +/-2,0%

# Labelling and certification schemes

The surfactant business is strictly B2B, i.e. surfactant producer sell their products only to companies making formulations for their respective markets, e.g. detergents, cleaner, textile auxiliaries, metal working fluids etc. Therefore, a consumer relevant labelling of surfactants is not necessary.

Certification of raw materials (regarding origin, sustainability etc.) along the value chain is important for surfactant producers and the consecutive users to gain labels, like Ecolabels (e.g. private ecolabel schemes as well as EU and national ecolabels schemes) for their finished goods, either for B2B or B2C business. Established certification schemes like RSPO (Round Table of Sustainable Palm oil [5]), ISCC (International Sustainability & Carbon Certification [6]), and others, for raw materials of bio-based surfactants can be used to demonstrate that.

It is also necessary to communicate the exact content of bio-based materials in bio-based surfactants to the surfactant users for their final calculations/measurements of their finished goods.  This can be determined either by the methods described in paragraph 6 of this report or by using methods like

  - EN 16785-2 Bio-based products – Bio-based content Part 2: Determination by material balance

Given by mandate M/491 [7], CEN/TC-411 developed standards for the communication between business partners and to consumers

  - EN 16848 Bio-based products – Requirements for Business to Business communication of characteristics using a Data Sheet

  - EN 16935 Bio-based products – Requirements for Business to Consumer communication and claims

Both standards are also suitable for the use regarding bio-based surfactants.

Bibliography

CEN/TS 17035 – Surface Active Agents - Bio-based surfactants - Requirements and test method

1272/2008/EC, Regulation (EC) No 1272/2008 of the European Parliament and of the Council of 16 December 2008 on classification, labelling and packaging of substances and mixtures, amending and repealing Directives 67/548/EEC and 1999/45/EC, and amending Regulation (EC) No 1907/2006. OJ EU. 2008, 353 (L) pp. 1–1355

      - EN ISO 14593:1999. Water quality. — Evaluation of ultimate aerobic biodegradability of organic compounds in aqueous medium. — Method by analysis of inorganic carbon in sealed vessels (CO2 headspace test). Pre-adaptation is not to be used. The ten day window principle is not applied

      - Method of the Directive 67/548/EEC Annex V.C.4-C [Carbon dioxide (CO2) Evolution Modified Sturm Test]: Preadaptation is not to be used. The ten day window principle is not applied.

      - Method of the Directive 67/548/EEC Annex V.C.4-E (Closed Bottle): Pre-adaptation is not to be used. The ten day window principle is not applied

      - Method of the Directive 67/548/EEC Annex V.C.4-D (Manometric Respirometry): Pre-adaptation is not to be used. The ten day window principle is not applied.

      - Method of the Directive 67/548/EEC Annex V.C.4-F (MITI: Ministry of International Trade and Industry-Japan): Pre-adaptation is not to be used. The ten day window principle is not applied.

1272/2008/EC, Regulation (EC) No 1272/2008 of the European Parliament and of the Council of 16 December 2008 on classification, labelling and packaging of substances and mixtures, amending and repealing Directives 67/548/EEC and 1999/45/EC, and amending Regulation (EC) No 1907/2006. OJ EU. 2008, 353 (L) pp. 1–1355

      - Method of the Directive 67/548/EEC Annex V.C.4-A (Dissolved Organic Carbon DOC Die-Away): Pre-adaptation is not to be used. The ten day window principle is not applied. The pass criteria for measured according to the test shall be at least 70 % within twenty-eight days.

      - Method of the Directive 67/548/EEC Annex V.C.4-B (Modified OECD Screening-DOC Die-Away): Pre-adaptation is not to be used. The ten day window principle is not applied. The pass criteria for biodegradability measured according to the test shall be at least 70 % within twenty-eight days.

2013/179/EU: Commission Recommendation of 9 April 2013 on the use of common methods to measure and communicate the life cycle environmental performance of products and organisations Text with EEA relevance. OJ EU. 2013, 124.

for details, please see www.rspo.org

for details, please see www.iscc-system.org

M/491, Mandate addressed to CEN, CENELEC and ETSI for the development of European Standards and Technical Specifications and/or Technical Reports for bio-surfactants and bio-solvents in relation to bio-based product aspects, 7 March 2011

A Mandate is a standardisation task embedded in European trade laws. M/492 Mandate is addressed to the European Standardisation bodies, CEN, CENELEC and ETSI, for the development of horizontal European Standards for bio-based products. M/491 mandate is addressed to the development of European standards for bio-solvents and bio-surfactants.

EN 16575

EN 16760

EN 16751