Surface active agents — Determination of average relative molecular mass of aliphatic alkyltrimethylammonium chloride —

Part 1:

GC method

Contents                                               Page

1 Scope	1

2 Normative references	1

3 Terms and definitions	1

4 Principle	1

5 Reagents and materials	1

6 Apparatus	2

7 Sampling	2

8 Procedure	2

9 Examination of chromatograms	3

10 Expression of results	3

11 Precision	4

12 Test report	4

Surface active agents — Determination of average relative molecular mass of aliphatic alkyltrimethylammonium chloride —

Part 1:

# GC method

- Scope

This document describes a Gas chromatography (GC) method for determination of the average relative molecular mass of aliphatic alkyltrimethylammonium chloride.

- Normative references

The following documents are referred to in the text in such a way that some or all of their content constitutes requirements of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ISO 607, Surface active agents and detergents — Methods of sample division

ISO 3696, Water for analytical laboratory use — Specification and test methods

- Terms and definitions

For the purposes of this document, the following terms and definitions apply.

ISO and IEC maintain terminology databases for use in standardization at the following addresses:

— ISO Online browsing platform: available at https://www.iso.org/obp

— IEC Electropedia: available at https://www.electropedia.org

- Principle

The aliphatic alkyltrimethylammonium chloride was thermally decomposed in the vaporization chamber of the gas chromatograph, and the average relative molecular mass of aliphatic alkyl dimethylamine was determined by the homolog distribution of the thermal decomposition products. The average relative molecular mass of aliphatic alkyl trimethylammonium chloride is calculated by adding the molecular mass of chloromethane (CH3Cl) to the average relative mass of aliphatic alkyl dimethylamine.

- Reagents and materials

During the analysis, use only reagents of recognized analytical grade and the water used shall conform to grade 3 in accordance with 3696.

5.1 Ethanol, 95%.

5.2 Reference aliphatic alkyltrimethylammonium chloride. For example, dodecyltrimonium chloride (1231), tedecyltrimonium chloride (1431), cetyltrimethylammonium chloride (1631), octadecyltrimonium chloride (1831).

5.3 Carrier gas: High purity nitrogen.

- Apparatus

Usual laboratory equipment, and

6.1 Gas chromatograph.

6.2 Chromatography column: Packed column, the column tube is made of inert material (such as glass or stainless steel), the inner diameter is 2mm~4mm, and the length is 0.5mm~1.5m; The support body is Chomosorb WAWDMCS or 405 silylated white support, with a particle size of 150μm~180μm, coated with 50%~10% potassium hydroxide and 5%~10% apiezon grease L (Apiezon L), which needs to be pre-aged before the experiment, and the chromatographic peaks of different carbon chain aliphatic alkyl dimethylamines can be well separated; or capillary columns, HP-5 or capillary columns with comparable performance.

6.3 Vaporization chamber, filled with silanized glass bead support (150μm~180μm).

NOTE  When using capillary chromatography columns, vaporization chamber may not be filled with silanized glass beads.

6.4 Chromatography workstation.

6.5 Syringes, 5μL or 10μL.

- Sampling

7.1 Preparation of the test sample

Prepare and store the test sample in accordance with ISO 607.

7.2 Preparation of test solution

Weigh about 0.5 g of the sample and dissolve it with a little water or ethanol (5.1) so that it can be aspirated with a syringe.

NOTE  Mass of the test sample can be adjusted depending on the detector sensitivity.

- Procedure

8.1 Apparatus settings

8.1.1 Vaporization chamber temperature: >300°C.

8.1.2 Column temperature: The initial temperature is 110 °C~130 °C, and the temperature is raised to the final temperature of 270°C~290 °C at a rate of 3 °C/min~5 °C/min.

8.1.3 Carrier gas flow: 30 mL/min ~ 50 mL/min.

8.1.4 Detector temperature: >300°C.

8.2 Performance inspection

Weigh about 0.2 g of reference samples (5.2) respectively and dissolve them with a little water or ethanol (5.1). A suitable amount of reference sample solution was injected into the chromatograph to make the chromatographic peaks of each carbon chain aliphatic alkyl dimethylamine well separated, and the retention time was recorded for comparison with the test sample.

NOTE  Mass of aliphatic alkyltrimethylammonium chloride can be adjusted depending on the detector sensitivity.

8.3 Determination

Inject an appropriate amount of sample (7.2) into the chromatograph with a syringe (6.5) so that the chromatogram peak height is appropriate. Typical chromatogram is shown in Figer 1.

- Examination of chromatograms

9.1 Qualitative analysis

Identify the components of the test portion by comparing the test chromatogram with that for the reference solution.

9.2 Quantitative analysis

Determine the area of each peak corresponding to a particular chain length. Calculate the total peak area.

- Expression of results

The average relative molecular mass M1 of the aliphatic alkyl dimethylamine is given by the formula (1):

where

Ai is the peak area of different carbon chain aliphatic alkyl dimethylamine;

A is the total peak area;

Mi is the relative molecular mass of different carbon chain aliphatic alkyl dimethylamine, given in Table 1.

Table 1 — The relative molecular mass

of the different carbon chain aliphatic alkyl dimethylamine

| The carbon chain | C12 | C14 | C16 | C18 |
| --- | --- | --- | --- | --- |
| Mi | 213.4 | 241.5 | 269.5 | 297.6 |

The average relative molecular mass M of the aliphatic alkyltrimethylammonium chloride is given by the formula (2):

Where

M1 is the average relative molecular mass of the aliphatic alkyl dimethylamine;

50.5 is he molecular mass of chloromethane (CH3Cl).

- Precision

11.1 Repeatability

The absolute difference between the results of two determinations carried out simultaneously or in rapid succession by the same operator on the same sample using the same equipment shall not be greater than 1 with a probability of 95 %.

11.2 Reproducibility

The absolute difference between two results obtained in different laboratories on the same sample shall not be greater than 2 with a probability of 95 %.

- Test report

The test report shall include the following information:

a) all information necessary for the complete identification of the sample;

b) a reference to this Standard;

c) the results with their units (see clause 9);

d) details of any operations not specified in this Standard or in the International Standards to which reference is made, and any operations regarded as optional, as well as any incidents likely to have affected the results.

Figer 1 — Gas chromatogram of the aliphatic alkyl dimethylamine

( a.Packed column. b. Capillary column)