ISO #####-#:####(X)

ISO TC 91/SC ##/WG #

Date: YYYY-MM-DD

表面活性剂  脂肪烷基三甲基氯化铵平均相对分子质量的测定

第2部分：

高效液相色谱法

WD stage

Warning for WDs and CDs

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

A model manuscript of a draft International Standard (known as “The Rice Model”) is available at https://www.iso.org/iso/model_document-rice_model.pdf

© ISO 20XX

All rights reserved. Unless otherwise specified, or required in the context of its implementation, no part of this publication may be reproduced or utilized otherwise in any form or by any means, electronic or mechanical, including photocopying, or posting on the internet or an intranet, without prior written permission. Permission can be requested from either ISO at the address below or ISO’s member body in the country of the requester.

ISO copyright office

CP 401 • Ch. de Blandonnet 8

CH-1214 Vernier, Geneva

Phone: +41 22 749 01 11

Email: copyright@iso.org

Website: www.iso.org

Published in Switzerland

Contents

Foreword	iv

Introduction	v

1 Scope	1

2 Normative references	1

3 Terms and definitions	1

4 Principle	1

5 Reagents and materials	1

6 Apparatus	1

7 Sampling	2

8 Procedure	2

9 Examination of chromatograms	2

10 Expression of results	3

11 Precision	3

12 Test report	3

Foreword

ISO (the International Organization for Standardization) is a worldwide federation of national standards bodies (ISO member bodies). The work of preparing International Standards is normally carried out through ISO technical committees. Each member body interested in a subject for which a technical committee has been established has the right to be represented on that committee. International organizations, governmental and non-governmental, in liaison with ISO, also take part in the work. ISO collaborates closely with the International Electrotechnical Commission (IEC) on all matters of electrotechnical standardization.

The procedures used to develop this document and those intended for its further maintenance are described in the ISO/IEC Directives, Part 1. In particular, the different approval criteria needed for the different types of ISO documents should be noted. This document was drafted in accordance with the editorial rules of the ISO/IEC Directives, Part 2 (see www.iso.org/directives).

ISO draws attention to the possibility that the implementation of this document may involve the use of (a) patent(s). ISO takes no position concerning the evidence, validity or applicability of any claimed patent rights in respect thereof. As of the date of publication of this document, ISO [had/had not] received notice of (a) patent(s) which may be required to implement this document. However, implementers are cautioned that this may not represent the latest information, which may be obtained from the patent database available at www.iso.org/patents. ISO shall not be held responsible for identifying any or all such patent rights.

Any trade name used in this document is information given for the convenience of users and does not constitute an endorsement.

For an explanation of the voluntary nature of standards, the meaning of ISO specific terms and expressions related to conformity assessment, as well as information about ISO's adherence to the World Trade Organization (WTO) principles in the Technical Barriers to Trade (TBT), see www.iso.org/iso/foreword.html.

This document was prepared by Technical Committee ISO/TC 91, Surface active agents.

Any feedback or questions on this document should be directed to the user’s national standards body. A complete listing of these bodies can be found at www.iso.org/members.html.

Introduction

一种测定脂肪烷基三甲基氯化铵平均相对分子质量的高效液相色谱法。该方法区别于气相色谱法的间接测定，将样品直接进样，在设定的色谱条件下对样品进行色谱分离，依据碳链分布，采用面积归一化法对样品的平均相对分子质量进行计算。采用F检验结合ｔ检验对该方法和QB/T 1914-2013描述的气相色谱法的测定结果进行统计分析，结果表明该测定方法与气相色谱法测定结果无显著性差异.

表面活性剂  脂肪烷基三甲基氯化铵平均相对分子质量的测定

第2部分:

高效液相色谱法

范围

本文件描述了一种高效液相色谱法（HPLC）测定脂肪族烷基三甲基氯化铵平均相对分子质量的方法。

规范性引用文件

本文档中引用了以下文件的全部或部分内容，这些文件对于本文档的应用是不可或缺的。对于有日期的引用文件，仅适用所引用的版本。对于无日期的引用文件，则适用引用文件的最新版本（包括任何修订）。

ISO 607，表面活性剂和洗涤剂——样品分样方法

ISO 3696，分析实验室用水——规范和试验方法

术语和定义

本文档中没有列出术语和定义。

国际标准化组织(ISO)和国际电工委员会(IEC)在以下地址维护用于标准化工作的术语数据库：

— ISO Online browsing platform: available at https://www.iso.org/obp

— IEC Electropedia: available at http://www.electropedia.org/

原理

采用高效液相色谱法对脂肪族烷基三甲基氯化铵进行色谱分离，并用蒸发光散射检测器进行测量。根据碳链分布，采用面积归一化法计算样品的平均相对分子质量。

试剂和材料

在分析中仅使用确认为分析纯的试剂和3696中规定的三级水。

5.1 乙腈, HPLC级, 经过滤装置过滤。

5.2 三氟乙酸, HPLC级, 经过滤装置过滤。

5.3 脂肪烷基三甲基氯化铵参考样品，如十二烷基三甲基氯化铵、十四烷基三甲基氯化铵、十六烷基三甲基氯化铵、十八烷基三甲基氯化铵。

仪器

常规实验仪器及

6.1 高效液相色谱仪.

6.2 蒸发光散射检测器 (ELSD)；

6.3 色谱柱，氰基柱 (250mm×4.6mm, 5μm)；

6.4 过滤装置, 0.45μm。

样品的制备

7.1 测试样品的准备

依据 ISO 607准备及储存测试样品。

7.2 测试溶液的制备

称取约0.1g样品至50mL容量瓶中，用乙腈 (5.1)溶解并定容, 用 0.45μm 的过滤装置(6.4)过滤。

注  可根据检测器灵敏度调整样品称样量.

程序

8.1 仪器条件的设定

8.1.1 流动相: 乙腈: 0.1mol/L 三氟乙酸=60:40

8.1.2 流速: 1,0 ml/min.

8.1.3 进样体积: 20 μl.

8.1.4 检测器: 蒸发光散射检测器 (ELSD)。根据使用的仪器及其物理参数优化工作条件。

8.2 校准

8.2.1 参考样品溶液的制备

分别称量0,05 g 1231、1431、1631及1831 (5.3) 至 50 ml 容量瓶中，用乙腈,(5.1)溶解并定容，经  0.45μm 的过滤装置(6.4)过滤。

注 可根据检测器灵敏度调整样品称样量。

8.2.2 参考样品色谱图

根据8.1中给出的色谱条件分析8.2.1中制备的对照品溶液。参考色谱图如图1所示。记录保留时间和获得的分离（以便将来与样品进行比较）。

8.3 测定

取7.2中制备的试验溶液，按照8.1中给出的色谱条件进行分析。典型色谱图如图2所示。

色谱图的检验

9.1 定性分析

用试样色谱图与参考样品色谱图对照，鉴别试样组分。

9.2 定量分析

测定不同碳链脂肪烷基三甲基氯化铵的峰面积，并计算总峰面积。

结果表示

脂肪烷基三甲基氯化铵的平均相对分子质量按公式(1)计算:

式中

Ai 不同碳链脂肪烷基三甲基氯化铵的峰面积；

A 总峰面积；

Mi 不同碳链脂肪烷基三甲基氯化铵的平均相对分子质量，见表1。

表 1 — 不同碳链脂肪烷基三甲基氯化铵的平均相对分子质量

| 碳链 | C12 | C14 | C16 | C18 |
| --- | --- | --- | --- | --- |
| Mi | 263.9 | 292.0 | 320.0 | 348.1 |

精密度

11.1 重复性

置信度为95%，相同样品，同一操作者用同一仪器两次相继测定所得结果的绝对差值不大于1。

11.2 Reproducibility

置信度为95%，同一样品，不同实验室间测定结果的绝对差值不大于2。

试验报告

测试报告应包括以下信息：

a） 完整识别样品所需的所有信息；

b） 对本标准的参考；

c） 结果及其单位（见第10条）；

d） 本标准或参考的国际标准中未规定的任何操作的详细信息，以及任何被视为可选的操作，以及可能影响结果的任何事件。

图 1 — 不同碳链的脂肪烷基三甲基氯化铵的参考色谱图

图 2 —  1631的色谱图