ISO #####-#:####(X)

ISO TC 91/SC ##/WG #

Date: YYYY-MM-DD

Surface active agents — Determination of average relative molecular mass of aliphatic alkyltrimethylammonium chloride —

Part 2:

HPLC method

WD stage

Warning for WDs and CDs

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

A model manuscript of a draft International Standard (known as “The Rice Model”) is available at https://www.iso.org/iso/model_document-rice_model.pdf

© ISO 20XX

All rights reserved. Unless otherwise specified, or required in the context of its implementation, no part of this publication may be reproduced or utilized otherwise in any form or by any means, electronic or mechanical, including photocopying, or posting on the internet or an intranet, without prior written permission. Permission can be requested from either ISO at the address below or ISO’s member body in the country of the requester.

ISO copyright office

CP 401 • Ch. de Blandonnet 8

CH-1214 Vernier, Geneva

Phone: +41 22 749 01 11

Email: copyright@iso.org

Website: www.iso.org

Published in Switzerland

Contents

Foreword	iv

Introduction	v

1 Scope	1

2 Normative references	1

3 Terms and definitions	1

4 Principle	1

5 Reagents and materials	1

6 Apparatus	1

7 Sampling	2

8 Procedure	2

9 Examination of chromatograms	2

10 Expression of results	3

11 Precision	3

12 Test report	3

Foreword

ISO (the International Organization for Standardization) is a worldwide federation of national standards bodies (ISO member bodies). The work of preparing International Standards is normally carried out through ISO technical committees. Each member body interested in a subject for which a technical committee has been established has the right to be represented on that committee. International organizations, governmental and non-governmental, in liaison with ISO, also take part in the work. ISO collaborates closely with the International Electrotechnical Commission (IEC) on all matters of electrotechnical standardization.

The procedures used to develop this document and those intended for its further maintenance are described in the ISO/IEC Directives, Part 1. In particular, the different approval criteria needed for the different types of ISO documents should be noted. This document was drafted in accordance with the editorial rules of the ISO/IEC Directives, Part 2 (see www.iso.org/directives).

ISO draws attention to the possibility that the implementation of this document may involve the use of (a) patent(s). ISO takes no position concerning the evidence, validity or applicability of any claimed patent rights in respect thereof. As of the date of publication of this document, ISO [had/had not] received notice of (a) patent(s) which may be required to implement this document. However, implementers are cautioned that this may not represent the latest information, which may be obtained from the patent database available at www.iso.org/patents. ISO shall not be held responsible for identifying any or all such patent rights.

Any trade name used in this document is information given for the convenience of users and does not constitute an endorsement.

For an explanation of the voluntary nature of standards, the meaning of ISO specific terms and expressions related to conformity assessment, as well as information about ISO's adherence to the World Trade Organization (WTO) principles in the Technical Barriers to Trade (TBT), see www.iso.org/iso/foreword.html.

This document was prepared by Technical Committee ISO/TC 91, Surface active agents.

Any feedback or questions on this document should be directed to the user’s national standards body. A complete listing of these bodies can be found at www.iso.org/members.html.

Introduction

This document describes a high-performance liquid chromatography method for determining the average relative molecular mass of aliphatic alkyl trimethylammonium chloride. This method is different from the indirect determination of gas chromatography in that the sample is injected directly, the sample is chromatographically separated under the set chromatographic conditions, and the average relative molecular weight of the sample is calculated by area normalization method according to the carbon chain distribution. The results of the HPLC method and the GC method described in QB/T 1914-2013 were statistically analyzed by F-test combined with t-test, and the results showed that there was no significant difference between the method and the gas chromatography.

Surface active agents — Determination of average relative molecular mass of aliphatic alkyltrimethylammonium chloride —

Part 2:

HPLC method

Scope

This document describes a High performance liquid chromatography (HPLC) method for determination of the average relative molecular mass of aliphatic alkyltrimethylammonium chloride.

Normative references

The following documents are referred to in the text in such a way that some or all of their content constitutes requirements of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ISO 607, Surface active agents and detergents — Methods of sample division

ISO 3696, Water for analytical laboratory use — Specification and test methods.

Terms and definitions

For the purposes of this document, the following terms and definitions apply.

ISO and IEC maintain terminology databases for use in standardization at the following addresses:

- ISO Online browsing platform: available at https://www.iso.org/obp

- IEC Electropedia: available at https://www.electropedia.org/

Principle

The aliphatic alkyltrimethylammonium chloride was chromatographically separated by HPLC and measured by evaporative light scattering detector. According to the carbon chain distribution, the average relative molecular mass of the sample was calculated by area normalization method.

Reagents and materials

During the analysis, use only reagents of recognized analytical grade and the water used shall conform to grade 3 in accordance with 3696.

5.1 Acetonitrile, HPLC grade, filtered with filter unit.

5.2 Trifluoroacetic acid, HPLC grade, filtered with filter unit.

5.3 Reference aliphatic alkyltrimethylammonium chloride. For example, dodecyltrimonium chloride (1231), tedecyltrimonium chloride (1431), cetyltrimethylammonium chloride (1631), octadecyltrimonium chloride (1831).

Apparatus

Usual laboratory equipment, and

6.1 High performance liquid chromatograph.

6.2 Evaporative light scattering detector (ELSD).

6.3 Chromatography column, Cyano column (250mm×4.6mm, 5μm).

6.4 Filter unit, for solvent (0.45μm).

Sampling

7.1 Preparation of the test sample

Prepare and store the test sample in accordance with ISO 607.

7.2 Preparation of test solution

Weigh, to the nearest 0.0001g, 0.1g of the test sample into a 50mL volumetric flask. Fill to the mark with acetonitrile (5.1), and filter through 0.45μm filter unit (6.4).

NOTE  Mass of the test sample can be adjusted depending on the detector sensitivity.

Procedure

8.1 Apparatus settings

8.1.1 Mobile phase: Acetonitrile: 0.1mol/L trifluoroacetic acid solution=60:40

8.1.2 Flow rate: 1,0 ml/min.

8.1.3 Injection volume: 20 μl.

8.1.4 Detector: evaporative light scattering detector (ELSD). Optimize the working conditions, depending on the apparatus in use and the physical parameters.

8.2 Calibration

8.2.1 Preparation of reference solution

Weigh respectively, to the nearest 0,1 mg, 0,05 g of dodecyltrimonium chloride (1231), tedecyltrimonium chloride (1431), cetyltrimethylammonium chloride (1631), octadecyltrimonium chloride (1831) (5.3) into the same 50 ml volumetric flask, dissolve with acetonitrile (5.1) and make up to the mark. Filter through a 0.45 μm filter unit.

NOTE: Mass of aliphatic alkyltrimethylammonium chloride can be adjusted depending on the detector sensitivity.

8.2.2 Reference chromatogram

Analyze reference solution prepared in 8.2.1, in accordance with the chromatographic conditions given in 8.1. The reference chromatogram is shown in Figure 1. Note the retention times and the separations obtained (for future comparison with the sample).

8.3 Determination

Take the test solution as prepared in 7.2 and carry out the analysis in accordance with the chromatographic conditions given in 8.1. Typical chromatogram is shown in Figure 2.

Examination of chromatograms

9.1 Qualitative analysis

Identify the components of the test portion by comparing the test chromatogram with that for the reference solution.

9.2 Quantitative analysis

Determine the area of each peak corresponding to a particular chain length. Calculate the total peak area.

Expression of results

The average relative molecular mass M of the aliphatic alkyltrimethylammonium chloride is given by the formula 1:

where

Ai is the peak area of different carbon chain aliphatic alkyltrimethylammonium chloride;

A is the total peak area;

Mi is the relative molecular mass of different carbon chain aliphatic alkyltrimethylammonium chloride, given in table 1.

Table 1 — The relative molecular mass of the homologues

| The carbon chain | C12 | C14 | C16 | C18 |
| --- | --- | --- | --- | --- |
| Mi | 263.9 | 292.0 | 320.0 | 348.1 |

Precision

11.1 Repeatability

The absolute difference between the results of two determinations carried out simultaneously or in rapid succession by the same operator on the same sample using the same equipment shall not be greater than 1 with a probability of 95 %.

11.2 Reproducibility

The absolute difference between two results obtained in different laboratories on the same sample shall not be greater than 2 with a probability of 95 %.

Test report

The test report shall include the following information:

a) all information necessary for the complete identification of the sample;

b) a reference to this Standard;

c) the results with their units (see clause 9);

d) room temperature for each liquid chromatographic determination and all information about ELSD ;

e) details of any operations not specified in this Standard or in the International Standards to which reference is made, and any operations regarded as optional, as well as any incidents likely to have affected the results.

Figer 1 — The reference chromatogram of different carbon chain aliphatic alkyltrimethylammonium chloride

Figer 2 — The chromatogram of 1631